C975 Flash-Flex Procedure for 80.2F.36.01P

NOTE: For experienced users only – contact Robert.Oskorep@motorola.com if you need help.

Requirement Summary:

- Use Motorola RSD Lite version 1.3 or higher program (see RSD Lite user guide and release notes if needed).

NOTE: RSD (Remote Software Download) Tool will be replacing the PST Tool. PST version 7.1.1 is last version of PST, and will eventually be discontinued. RSD Lite is similar to PST MultiFlash-Flex program, but it is not the same. There will also be a RSD General program released later that can backup and re-store phone data as well as upgrade software.

- PC with Microsoft Windows 2000 or higher (see RSD Lite user guide and release notes if needed)

Step 1:

Flash in the appropriate flash and flex (1FF) combination file using RSD Lite 1.3 Program

R24_U_80.2F.36.01P_LP002C_DRM0101_JPAU_DAP_R24_80.2F.36.01P_SAC975T672WJ00F_1FF.shx

Step 2:

Perform Master Clear and then Master Reset in User Menu

Step 3:

Verify Software version in the handset through user menu:

Menu> Settings>Phone Status>Other Information

Software version: R24_U_80.2F.36.01P

Flex version: SAC975T672WJ00E

DSP S/W 40450000

: