## Bluetooth

Test Description

The purpose of this test procedure is to verify that the Bluetooth module is connected and functioning properly.

| Test Dependencies: Test Channels (USA) |  |
| --- | --- |
| Channel A | 39 |
| Table 1.1 |  |

N/A

Equipment:

Bluetooth Test Set or equivalent

The test channel in table 1.1 is for measuring BER or Power only. Otherwise, the appropriate frequency-hopping pattern must be utilized.

Test Setup

Bluetooth test set setup:

Frequency Hopping	EUSA (OFF for BER and Power tests)

Number of packets	100

Test type		LOOPBACK (For BER test only. TX / RX for all others)

Packet type		DH1

Byte Length		25 bytes

Pattern			Static PRBS

Whitening		On

Master TX Level		-70 dBm	(at the UE)

Channel			Channel A (see table 1.1)

Connect the Bluetooth test set to an antenna fixture approx 1” away from the UE.

For the Bluetooth Connection Test, the Bluetooth RF path must be padded with enough attenuation such that the Bluetooth signal from the UE to the test set is 15 dB (±5 dB) above the measured input sensitivity level of the test set.

Note: This path attenuation may not be suitable for Bluetooth Power measurements.

If the Bluetooth Connection, BER or Power tests are to be performed, the RF path must be free from interfering signals in the 2400 to 2483.5 MHz frequency range. WLAN channels 3 through 9 will cause interference in this frequency range. A minimum of 80 dB of attenuation on the interferer is required for these tests.

The Bluetooth module must be in the OFF state at the start of this procedure. Use the BT command to turn the Bluetooth module OFF (if necessary) and download the Bluetooth diagnostic firmware.

Command	Opcode/data

BT		0x0042 86		(Power off Bluetooth IC)

BT		0x0042 84		(Download Diagnostic Firmware)

BT		0x0042 81		(Place Bluetooth stack into HCI test mode)

BT		0x0042 0103 1800	(HCI Enable Device)

(Continued on next page)

Test Procedure

Bluetooth Host and Wake Test:

Poll the UE for an unsolicited response for up to 21 seconds. Verify that the unsolicited response data = 0x8400.

Send the following test command to the UE.

Command	Opcode/data

RDWR_IO	0x0C25 0000 07				(Read Bluetooth host wake line)

Poll the RDWR_IO response for up to 15 seconds until the response = 0x01.

Send the following test commands to the UE.

Command	Opcode/data

RDWR_IO	0x0C25 0100 0800		(Set Bluetooth wake line low)

RDWR_IO	0x0C25 0000 07			(Read Bluetooth host wake line)

Verify that the response = 0x00.

Send the following test commands to the UE.

Command	Opcode/data

RDWR_IO	0x0C25 0100 0801		(Set Bluetooth wake line high)

RDWR_IO	0x0C25 0000 07			(Read Bluetooth host wake line)

Verify that the response = 0x01.

Bluetooth Audio Loopback:

Send the following commands to the UE.

Command	Opcode/data

BT		0x0042 011C FC04 0002 0000 00			(Set PCM mode)

BT		0x0042 011E FC05 0000 0103 01			(Set PCM data format)

BT		0x0042 0122 FC01 01				(Set PCM Time Slot 1)

BT		0x0042 0124 FC01 01				(Enable audio loopback in the BT module)

AUD_PATH	0x0006 7B00					(Bluetooth audio path TS1)

AUD_TN_GEN	0x0044 8100 5A83 7FFF 0000 0000 0000 0000	(Generate 1kHz tone)

Wait 100ms

AUD_SAMP	0x0C15 0814					(Read looped audio)

Preferred verification:

Take an FFT of the returned data. Verify that the largest frequency component of the returned signal is 1 kHz. Verify that there are no other major frequency components present in the returned signal.

Alternate verification:

Analyze the returned response. Verify that the returned data does NOT contain all 0’s.

Send the following commands to disable audio loopback.

Command	Opcode/data

BT		0x0042 0124 FC01 00				(Disable Bluetooth audio loopback)

AUD_TN_GEN	0x0044 0100 5A83 7FFF 0000 0000 0000 0000	(Stop tone generator)

(Continued on next page)

Bluetooth CW Power Out:

Send the following commands to the UE:

Command	Opcode/data

BT		0x0042 010A FC04 0300 5500		(Turn Off TCA)

BT		0x0042 0130 FC01 02			(Set PA Bias)

BT		0x0042 012D FC01 29			(Enable a CW signal at 2441MHz)

Measure the power at 2441 MHz (Channel 39). Record this value as Bluetooth CW Power.

Verify that Bluetooth CW Power is within the limits specified in section Error: Reference source not found Error: Reference source not found.

Bluetooth Connection Test:

Determine a unique 6-byte BD address.

Send the following commands to the UE.

Command	Opcode/data

BT		0x 0042 0103 0C00				(HCI Reset)

BT		0x0042 81		                                           (Place Bluetooth stack into HCI test mode)

BT		0x0042 0103 1800	                                           (HCI Enable Device)

BT		0x 0042 010A FC04 0300 5501			(Turn On TCA)

BT		0x0042 0101 FC06 + Address (byte-swapped)	(Set Unique BD Address)

BT		0x0042 011B FC01 01				(Set external equipment as master)

BT		0x0042 011A 0C01 03				(Make local device discoverable)

BT		0x0042 0105 0C03 0200 02			(Auto accept incoming connection)

Connect to device.

Note: Allow up to 5 seconds to establish the connection.

Verify that a connection is established successfully.

Bluetooth Power & BER:

Measure TX Power while the device is connected. Record this value (Bluetooth TX Power).

Measure BER while the device is connected. Record this value (Bluetooth RX BER).

Verify that Bluetooth TX Power and Bluetooth RX BER within the limits specified in section Error: Reference source not found Error: Reference source not found.

Disconnect the Bluetooth device.

Disable Bluetooth.

Command	Opcode/data

BT		0x0042 86					(Terminate Bluetooth)

| Parameters | Lower Limit | Upper Limit | Unit |
| --- | --- | --- | --- |
| Bluetooth Host and Wake Bluetooth Audio Verify Bluetooth CW Power Bluetooth Connection Bluetooth TX Power Bluetooth BER | 1 1 -10 1 -10 0 | 1 1 +10 1 +10 2 | P/F P/F P/F dBm % |