Service Manual

S6 GSM / PCN

V 1.3

Total number of pages: 31

# Table of Contents

1 Table of Contents

2 Technical Data

3 General Information

4 Mechanical Concept

4.1 S6 GSM / PCN mechanical drawings

4.2 Disassembling the S6

4.3 Assembling the S6

4.4 Board Versions

4.5 Upper Case Protection

4.6 Handset Datecodes

5 Hardware Concept

5.1 Block Diagram

5.2 Hardware Description

5.3 Power Supply Concept

5.4 Overvoltage Conditions

6 S6 GSM / PCN Software Options

7 Software programming

7.1 Description of software booting

7.2 Language Groups

8 Deblocking

9 Battery

9.1 Specification

9.2 Short Circuit Protection

9.3 Deep Discharge

9.4 Battery Datecodes

10 S6 GSM / PCN Accessories

10.1 Desktop Charger

10.2 Car Charger

10.3 Car Kit Basic

10.4 Car Kit Handsfree

11 Spare Parts List

11.1 swap units

11.2 spare parts

11.3 customer spezified spare parts

11.4 Accessories and wearing parts

11.5 Documentation

# Technical Data

Length 159 mm

Width 55 mm

Thickness 16/22 mm

Volume 155 cm³

Weight 165 g

Performance:	GSM class 4 (2 Watt)

PCN class 1 (1 Watt)

Powersupply: Lithium Ion battery (3,6V/1000mAh)

Standby time: up to 30 hours

Talk time: Up to 4 hours.

Carging Time: Full charge in 2.5 hours

Display LCD high-resolution graphic display, 97 x 33 pixels, 4 rows of 16 characters

SIM Card Type: Small (plug in) 3V or 5V

# General Information

The S6 is the first model of a whole new handset generation in 3 Volt technology.
It comes either as a GSM or a PCN version.

# Mechanical Concept

Note: All part numbers refer to mechanical drawing in section 4.1!

The S6 consists of two boards, the RF & Control module (1010) and the user interface (1020).

The connection between those two boards is not made by a normal connector with plug-in contacts, but by a special interconnector (1090) embedded into a shielding frame (1060). This interconnector is upholding the connection through the pressure implied on it by the housing.

Be careful when assembling the interconnector. Avoid any kind of dust or dirt because it will affect the contacts of the interconnector.

The only removable part on the user interface is the ringer gasket (1080) which is needed to increase the ringer volume.

Furthermore there is a RF-plug on the user interface which has to be plugged into the appropriate connector on the RF & control module. This conection is very essential because it is the RF signal from / to the internal antenna.

The shielding is done by the lower case shell (1050) which is covered with a conductive material, so no additional shielding screen is necessary.

The antenna (1130) is directly screwed into the lower case shell.

The keypad (1030), the loudspeaker (1120), the microphone (1110) and the dust protection frame (1210) are mounted into the upper case shell (1040). Make sure that the microphone contacts are properly bent up when mounting.

When turning in the screws (1140, 1145 and 1150) make sure that the right torque is used (25 Ncm), because this will have an effect on the contacts of the interconnector.

Note:	Make sure that you mount the right antenna, the GSM antenna has two 
	stripes, the PCN antenna only has one!

Figure 1: Mechanical Drawing S6

## S6 GSM / PCN mechanical drawings

Figure 2: Display with Dust Protection Frame

Note: The numbers displayed in the above drawings are NO ORDERING Nos, 
           use the numbers in chapter 11 for ordering!

Figure 3: Dust Protection Frame

Ordering number for dust protection frame:

L36859-A4-C141

## Disassembling the S6

ESD regulations have to be followed!

- Remove battery and SIM-Card.

- Unscrew antenna (1130).

- Unscrew 2 oval head screws (1150) (Torx 1,8x12).

- Turn out 2 short flat head screws (1140) (Torx 1,8x8).

- Remove plastic disks (1155) and turn out the 3 long flat head screws (1145) 
	(Torx  1,8x12).

- Lift off the lower case shell (1050).

- Lift composite of MMI-board (1020) and RF-and controle module (1010) out 	of the upper case shell (1040).

	Only for level-2 repairs:
		
		a) Remove ringer gasket (1080) from the MMI board (1020).
		b) Remove RF plug from RF- and control module (1010).
		c) Take off the MMI board (1020).
		d) Take frame (1060) off the RF- and control module (1010).

- e) Remove interconnector (1090) from the frame (1060).

- If necessary: Take battery connector out of lower case shell (1050).

- Remove the dust protection frame (1210) from the upper case shell (1040).

- Take keypad (1030) out of upper case shell (1040).

- Remove earphone capsule (1120) and microphone (1110) from upper case 	shell.

## Assembling the S6

ESD regulations have to be followed!

- Put earphone capsule (1120) and microphone (1110) into upper case shell.

- Insert keypad (1030) into upper case shell (1040).

- Put dust protection frame (1210) on display window inside upper case shell.

- If necessary: Insert battery connector into lower case shell (1050).

- Insert composite of MMI-board (1020) and RF-and controle module (1010) into 	upper case shell (1040).

	BEFORE (only for level 2 repairs):

		a) Put interconnector (1090) to the frame (1060).
		b) Positioning of frame (1060) on the RF- and control module (1010).
		c) Place MMI board (1020) on top of frame (1060).
		d) Connect RF plug of MMI board (1020) to RF- and control module (1010).
		e) Mount ringer gasket (1080) on the MMI board (1020).

- Close device with upper case shell (1040). Make sure that the 6 hooks of the 	upper case shell fit firmly into the respective slots of the lower case shell (1050).

- Screw 3 long flat head screws (1145) (Torx 1,8x12 /Torqe 25 Ncm) into lower 	case shell (1050). Close holes with plastic disks (1155) afterwards.

- Screw 2 short flat head screws (1140) (Torx 1,8x8 /Torqe 25 Ncm) into lower 	case shell.

- Screw 2 oval head screws (1150) (Torx 1,8x12 /Torqe 25 Ncm) into lower case 	shell.

- Screw in Antenna (1130) (Torqe 25 Ncm).

- Insert SIM-Card and battery.

## Board Versions

In the S6 mechanical drawing you see a bolt (position 1220) which is holding the RF&control module and the user interface together. This bolt was mounted only in the first 100k S6 handsets produced. 
The handsets produced later are using different board versions with different part numbers (see table below!)

If a bolted handset comes back for service and one of the boards is defective, do not try to separate them. Just send the whole sandwich back to Germany.

Attention!

These numbers are not the spare part ordering numbers, they are manufacturing numbers printed on the boards itself.

Do not use them for ordering spare parts!

## Upper Case Protection

If the bolted boards are used, it has to be made sure that one guiding pin in the upper case shell is removed (see drawing!).

If the pin is not removed, the case may break!

Attention!

Do not remove the pin if the unbolted boards are used!

The removal of the pin is simply done by cutting it off with a sharp knife or cutter.

## Handset Datecodes

Siemens is using the industrial standard DIN EN 60062 to indicate the production / service dates. The code is printed on the IMEI sticker located under the simcard reader.

-> YY = Datecode

The first character of the datecode indicates the year of production:

F = 1995

H = 1996

J = 1997

The second character indicates the month of production:

1-9 = january to september

O = october

N = november

D= december

Example: HD means that the set was produced in december of 1996.

# Hardware Concept

## Block Diagram

## Hardware Description

The handset consists of five major integrated circuits:

- GOLD-µC (PMB 2706)

This microprocessor is responsible for controlling the keyboard, SIM-Card, EEPROM, Flash and RAM.
Furthermore it interfaces the GOLD-SP (PMB 2707), controls the power up/power down of the RF module and sets the amplification of the PA.

- GOLD-SP (PMB 2707)

This signal processor is responsible for processing the Rx I/Q signals (filtering, equalizing, speech and channel decoding).
Furthermore it does the speech and channel encoding and the GSMK modulation of the Tx I/Q signals.

3) GAIM (PMB 2905) 

The GAIM provides the interface between the analogue signals (I/Q, voiceband, PA-control) and its digital representation.

4) Receiver Circuit (PMB 2405/07)

This circuit provides the following main functionalities:

a) Low Noise Amplifier (LNA) with a fixed amplification of +20dB to amplify the input 
    RF signal.

b) Mixer to mix down the RF signal to the Intermediate Frequency (IF)

c) Programmable IF amplifier with a dynamic range of 60dB ( -10dB ... +50dB in 
    steps of 2dB)

d) Mixer to mix down the IF signal to the baseband, generating and inphase (I) and a 
    quadrature (Q) signal.

e) Offset compensation for the I/Q signals.

5) Transmitter Circuit PMB 2240/45

This circuit provides the IF synthesizer, the I/Q modulator, prescalers to regulate the RF synthesizer and a buffer stage to feed the PA.

The antenna switch is mechanical, whenever you plug in the bottom connector, the handset will switch to its external antenna.

TH

## Power Supply Concept

The S6 has two main power inputs (see Blockdiagram):

1) Battery Voltage (3.6 Volts) connected at the battery contacts

2) Charging Voltage (6.5 Volts) delivered by

a) The plug-in charger at the charging plug

b) The desktop charger (using the charging contacts at the bottom of the phone)

Since the battery voltage is supplying the power supply asic, it is always needed to operate the phone. You cannot switch on the handset if the battery voltage is not present.

From the 3.6 V battery voltage, all other supply voltages of the S6 are derived, controlled by the power supply ASIC.
The RF module needs 6.4 V for its PA, this voltage is generated by a step-up converter.

The logic module uses 2.8 V, generated by a simple linear regulator.

Furthermore the ASIC generates the supply voltage for the SIM-Card and the RESET signal for the logic devices.

The ASIC also checks the presence of the watchdog signal from the µP and provides the switching on functionality (ON_OFF button or Ignition signal).

During testing ist is advisable to use a battery dummy, connected to a power supply delivering +4V, max 3A.

Make sure that you connect the battery dummy with the right polarity, the red plug to +4V and the blue plug to ground.

If you use a voltage higher than +7V, or with wrong polarity, the phone can be destroyed!

## Overvoltage Conditions

a) Battery Voltage:		If the battery voltages rises above 6.2 Volts, the phone will

switch off and it cannot be switched on again before the 				voltage is lower than 6.2 Volts.

If the battery voltage rises above 7 Volts the phone can be

destroyed.

b) Charging Current:	The charging current must not rise above 1 A or the 					phone (fuse) will be inoperable.

-> Be careful with foreign accessories or chargers!

-> Make sure that the charging current is limited to a value below 1A!

# S6 GSM / PCN Software Options

The following list will help you to understand what the different software options
are for and how they are used.

Keep in mind that many of these features are network dependent, a malfunction 
cannot automatically be blamed on the handset.

OPTIONS

- RINGER

  - Ringer 	On / Off
The ringer can be switched on or off

  - Volume 	Adjust  
The ringer volume can be adjusted

  - Tone 	Adjust
The ringer tone can be adjusted

- DIVERT

  - If not Reachable	Clear / Set / Check
	Divert when the phone is switched off

  - If no Reply	Clear / Set / Check
	Divert if nobody answers the call

  - All Calls   	Clear / Set / Check
	Divert all incoming calls

  - If Busy	Clear / Set / Check
	Divert if the line is busy

- CONTROL USE

  - Key Lock
	Lock your keys from unwanted pressing.
	Unlock by holding down the „#“ button.

  - Pin Control	Alter / On / Off
	Some networks allow you to swith off your PIN control

  - Allow only *&	Set / Clear          	PIN2
	
	This will limit all outgoing calls to the numbers stored in the
	FDN - Phonebook (FDN = Fixed Dialling Numbers), which is only
	available in a phase 2 SIM-Card.
	
	
	If you are using a phase 1 SIM-Card you will see the following:
	Allow only   &	Set / Clear	PHONECODE
		
	This will limit all outgoing calls to the numbers stored in the normal
	Phonebook.

  - Allow last 1-5	Set / Clear	PHONECODE
	This will limit all outgoing calls to the numbers
	stored in the redail list (maximum five numbers!)

  - Call Screening	Set / Clear	PHONECODE
	This feature will filter the incoming calls. If a call comes in which
	number is stored in the phonebook, the phone will ring.
	Otherwise the phone will only give an indication on the display.

  - SIM Lock

    - Prevent new SIM	Set / Clear	PHONECODE
	If you set this feature, the phonecode will 
	be activated.
	Whenever another SIM-Card is inserted,
	the phone will ask for the phonecode.

    - Network Lock	Set / Clear  	NETWORKCODE
 	If you have a network lock activated,
	the phone will only accept SIM-Cards
	of this network operator.

    - Service-P. Lock	Set / Clear	SERV.-P. CODE
	If you have a service provider lock 
	activated, the phone will only accept 
	SIM-Cards of this service providers.

  - Loan Phone	Set / Clear	PHONECODE
	If you set this feature, the „Prevent new SIM“
	feature will be activated, and you cannot change
	the charging information anymore.

  - Network Barring

    - All Calls Out	Set / Clear	PASSWORD
No outgoing calls possible

    - Out Internat.	Set / Clear	PASSWORD
Just national calls allowed

    - Out Int.excHome	Set / Clear	PASSWORD
No international calls except to the home PLMN

    - All Calls In	Set / Clear	PASSWORD
No incoming calls possible

    - When Roaming	Set / Clear	PASSWORD
No incoming or outgoing calls when roaming

- MESSAGES

  - Voice Message	Alter / Hear
	You can save the telephone number of your voice
	mailbox to hear your messages

  - .....Text ( x old )
	Read your received SMS messages

  - Send Text
	Send a SMS message

  - ......Known Calls
	Telephone numbers of calls not accepted.
	Only available if CLIP is active!

- GSM (PCN) SERVICE

  - Network Info
	This feature will show you all available and accessible
	GSM (PCN) networks

  - Reselect Netw.
	Restart the network searching process

  - Auto Network	On / Off
	If you switch on auto network the mobile will first
	try to find the home network, then the networks in
	the network list and finally the strongest available

  - Network List
	Free programmable priority list for networks

  - Call Waiting	Check / On / Off
	The second call coming in will be indicated (knocking)

  - Conceal ID	Check / On / Off
	Your number will not be presented to the called party (CLIR)

  - Power Save Mode	On / Off
Periodic location update will be delayed to save energy

- TIME CHARGE

  - Last Call
	Length and cost of last call

  - Total Calls	Clear              (PHONECODE)
	Lenght and cost of all calls

  - Auto Display	On / Off
	Automatic lenght and cost display

  - 1 Minute Beep	On / Off
	1 minute indication during call

  - Charge Rate
	Setting of charge rate

  - Charge Limit	On / Off          (PHONECODE / PIN 2)
	Setting and activating of charge limit 
	If you have a phase 1 SIM-Card you will have to enter
	the phonecode, with a phase 2 SIM-Card the PIN 2 is
	needed.

- LANGUAGE

  - Automatic
	Language which preferred in the SIM-Card is used,
	if the language is in the mobile software.

  - English

  - Deutsch

  - Francais

- CAR USE

  - Auto Answer	On / Off
	If a call comes in it will be automatically answered

  - Auto Power Off
	You can set the time of auto power off after ignition
	has been switched of.

- SET UP

  - DTMF Key Tone	On / Off
	DTMF for remote control of answering machines etc.

  - Key Sound	Click / Tone / Silent

  - Any Key Answer	On / Off
	Any key will hook off

  - Illumination	On / Off
	Switch off illumination permanently to save energy

  - Own Greeting	On / Off
	Program your own greetings text

  - Service Tones	On / Off
	Service tones are audible when loggin into the net-
	work or connecting a call etc.

  - Master Reset
	This feature will reset factory settings, but not the se-
	curity codes or charge limits etc.

  - Phone Status
	This shows your IMEI-Number and the Software-Date
	(Press left softkey once!)

# Software programming

The software of the S6 GSM / PCN handsets is programmed directly from a PC using the bootadapter (see drawing below).

## Description of software booting

Connect COM-port of PC with bootadapter using the enclosed RS232 cable.

Afterwards plug in AC-Adapter, if connected correctly the „Power“ lamp will be active.

Switch off handset and connect it to the bootadapter with the connection cable.

Copy bootsoftware to your PC and follow the instructions in the „readme.txt“.

Ordering Number Bootadapter:	L24857-F1006-A30

The bootadapter comes complete with AC-Adapter, RS-232 and handset connection cable.

## Language Groups

There are different language groups of the same software version:

Attention: This information is subject to change! Contact your service coordinator for the latest update and ordering numbers.

a) GSM

| Language Group | Display Languages |
| --- | --- |
| 1 | German, English, French, Italian |
| 2 | Danish, Finish, Swedish, Norwegian |
| 3 | English, Katalanish, Spanish, Portuguese |
| 4 | English, Turkish, Czech, Polish |
| 5 | English, Russian, Bulgarian, German |
| 6 | English, Greek, Italian, Hungarian |
| 7 | English, French, Dutch, SMS-CB |
| 8 | English, French, Arabic, German |

b) PCN

| Language Group | Display Languages |
| --- | --- |
| 1 | German, English, French, SMS-CB |
| 2 | Danish, Finish, Swedish, Norwegian |

# Deblocking

If the phone is disabled due to a wrong entry of the phonecode (not PIN1, PIN2, network code or service provider code!) it can only be resetted by entering the right unblocking code.
The unblocking code is derived from the IMEI number and can only be calculated by our hotline personell in Germany.

If you need unblocking codes just send a fax with the IMEI numbers to:

Siemens AG

PN KE SH

World Service Center

Bocholt, Germany

Fax: +49 2871 91 3007

Please use the appropriate form provided by your service coordinator!

# Battery

## Specification

The S6 battery is a Lithium-Ion type with a voltage of

3.6 Volts and a capacity of 1000 mAh.

The connections BATT+ and GND are used to supply the mobile, while RCODE is used to detect the battery technology. BATT_TEMP is used to measure the battery temperature.

## Short Circuit Protection

The battery is short-circuit protected by an electronic fuse.

The resetting of the fuse can be done by the following procedures:

* Plug the battery into the desktop charger.

or

* Apply +5 Volts between the BATT+ and GND contacts (see drawing!)

Limit the current to 10mA.

## Deep Discharge

If the battery is deeply discharged it can be recharged by the following procedure:

Insert flat battery into handset and connect travel charger. The charging symbol will

not be visible. Wait for appr. 1 hour and disconnect charger afterwards.

Remove battery and reinsert it. If you connect the travel charger now, the charging
symbol must be visible on the handset display.

## Battery Datecodes

The datecode printed on the battery will give you the following information:

Example: TOS 8 G9 VA 1

TOS = Cells supplied by Toshiba

8 = Revision level

G9 = Datecode

VA = Varta (battery manufacturer)

1 = Place of manufacturing (1=Germany ,2= Novibor, Czech Republic)

Datecode:

1. Character: Year of production (F= 1995, G= 1996)

2. Character: Month of production (1-9 means January to September; O=October; N=November ; D=December)

From february of 1997 on, Varta is going to adapt its datecoding according to industrial standard DIN EN 60062.

This means that the character indicating the year 1997 will be a „J“

Example: 	TOS 8 J2 VA 1	This battery was produced in february of 1997!

# S6 GSM / PCN Accessories

Depicted are the following accessories:

## Desktop Charger

This charger can be used to charge the battery alone or the battery inserted into the handset, but not both at the same time!

Furthermore ist is helpful to reset disabled batteries.

The desktop charger is supplied by the AC-adapter coming with the handset.

## Car Charger

The car charger is connected to the cigarette lighter of the car with a flexible cable.

This accessory will charge the handset battery, it can not be used as a car kit because it does not have a RF connection to the external antenna.

Note: The cigarette lighter plug is only usable for 12V car batteries.
	Do not connect it to a 24V truck battery!

Never cut off the cigarette lighter plug and never make a direct connection to 	the car battery. There is a voltage regulator in the plug which is regulating

the 12V down to the charging voltage of the handset!

## Car Kit Basic

The car kit basic is connected to the cigarette lighter of the car with a flexible cable.

This accessory will charge the handset battery, and it can also be used as a car kit because it does provide a RF connection to the external antenna.

You have to use a external antenna because the phone automatically switches off the internal antenna!

Note: The cigarette lighter plug is only usable for 12V car batteries and for 24V truck 	batteries!

Never cut off the cigarette lighter plug and never make a direct connection to 	the car battery. There is a voltage regulator in the plug which is regulating

the 12V/24V down to the charging voltage of the handset!

## Car Kit Handsfree

This car kit provides the full range of functionalities: handsfree microphone and loudspeaker, battery charging and radio mute.

The connections to the battery are made directly using the cables with fuses for „battery+“, „battery-“ and „ignition“. Furthermore it provides a „radio mute“ connection to the car radio.

See drawing „S6 GSM / PCN Accessories“ for details-

# Spare Parts List

List of spare parts / jigs and tools / swap units

Attention: The following lists are subject to change. Contact your service coordinator for the latest update!

## swap units

These are complete telephones for swap-purposes.

| Part-Number | Part | Q.p.P. |
| --- | --- | --- |
| L24859-C2600-A910 | S6/GSM/SWAP/LG 1 | /1 |
| L24859-C2600-A914 | S6/GSM/SWAP/TIM/LG 1 | /1 |
| L24859-C2600-A915 | S6/GSM/SWAP/OPI/LG 1 | /1 |
| L24859-C2600-A917 | S6/GSM/SWAP/TDM/SIM-LOCK | /1 |
| L24859-C2600-A922 | S6/GSM/SWAP/MOVISTAR/SIM-LOCK | /1 |
| L24859-C2600-A923 | S6/GSM/SWAP/AIRTEL/SIM-LOCK | /1 |
| L24859-C2600-A924 | S6/GSM/SWAP/SIEMENS/LG 7 | /1 |
| L24859-C3600-A910 | S6/PCN/SWAP/LG 1 | /1 |
| L24859-C3600-A914 | S6/PCN/SIM-LOCK/ADAM/SWAP/LG 1 | /1 |
| L24859-C3600-A915 | S6/PCN/SIM-LOCK/MUTIA/SWAP/LG 2 | /1 |

LG = Language Group

## spare parts

|  | Part.-No. | Parts | Q.p.P. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | L24859-A2600-A905 | S6/GSM/RF/CONTROL/BOARD/ LG1 | /1 |  |  |  |  |
|  | L24859-A3600-A905 | S6/PCN/RF/CONTROL/BOARD/LG1 | /1 |  |  |  |  |
|  | L24851-F1000-A20 | CONNECTION-BOX/+LOUDSPEAKER |  | /1 |  |  |  |
|  | L36158-A4-B2 | LOWER/CASE/SHELL/STD/COMPLETE | /20 |  |  |  |  |
|  | L36158-A4-C8 | FOAM/SEAL/BUZZER | /10 |  |  |  |  |
|  | L36158-A4-C11 | COVER/RUBBER/SCREW | /25 |  |  |  |  |
|  | L36158-A4-C27 | SCREEN/FRAME | /10 |  |  |  |  |
|  | L36212-Z3-C15 | ACOUSTIC/EARPHONE | /10 |  |  |  |  |
|  | L36254-Z6-C59 | MICROPHONE/GSM/PCN | /10 |  |  |  |  |
|  | L36334-Z93-C253 | INTERCONNECTOR | /10 |  |  |  |  |
|  | L36851-Z1361-B18 | BATTERY CONTACT 4pins | /40 |  |  |  |  |
|  | L36851-Z1402-A10 | KEYPAD/GSM/PCN | /10 |  |  |  |  |
|  | L36851-Z1508-A17 | S6/GSM/USER INTERFACE | /1 |  |  |  |  |
|  | L36851-Z1508-A18 | S6/PCN/USER INTERFACE | /1 |  |  |  |  |
| ## customer spezified spare parts |  |  |  |  |  |  |  |
|  | L36158-A4-B1 | UPPER/CASE/SHELL/GSM/STD | /20 |  |  |  |  |
|  | L36158-A4-B11 | UPPER/CASE/SHELL/PCN/STD | /20 |  |  |  |  |
|  | L36158-A4-B22 | UPPER/CASE/SHELL/GSM/TDM | /20 |  |  |  |  |
|  | L36158-A4-B28 | UPPER/CASE/SHELL/GSM/TIM | /20 |  |  |  |  |
|  | L36158-A4-B29 | UPPER/CASE/SHELL/GSM/OPI | /20 |  |  |  |  |
|  | L36158-A4-B31 | UPPER/CASE/SHELL/GSM/MOVISTAR | /20 |  |  |  |  |
|  | L36158-A4-B32 | UPPER/CASE/SHELL/GSM/AIRTEL | /20 |  |  |  |  |

## Accessories and wearing parts

| Part.-No. | Parts | Q.p.P. |  |
| --- | --- | --- | --- |
| L24158-Z27-A308 | SCREW/FLAT/HEAD/TORX 1,8x8 |  | /40 |
| L24851-Z2705-A20 | BATTERY/PACK/LITHIUM-ION |  | /1 |
| L36146-A1016-D | CON.CABLE/BATTERY |  | /1 |
| L36158-A4-C40 | BELT CLIP |  | /10 |
| L36158-A5-B1 | HOLDER/HANDSET |  | /1 |
| L36158-Z27-A212 | SCREW/FLAT/HEAD/TORX 1,8x12 |  | /40 |
| L36158-Z28-A312 | SCREW/OVAL/HEAD/TORX1,8x12/HANDSET/ *) |  | /40 |
| L36254-Z6-C61 | HANDSFREE/MICROPHON/GSM/PCN |  | /1 |
| L36851-Z1303-A134 | CON.CABLE/CARKIT/HF |  | /1 |
| L36158-A2-C5 | DISTANCE/STRIP *) |  | /1 |
| L36158-A5-C4 | PLUG-IN/CONSOLE *) |  | /1 |
| L36158-A5-C5 | PLUG-IN/COVER *) |  | /1 |
| L24851-Z2630-A2 | DESK-TOP/CHARGER #) |  | /4 |
| L36851-Z2618-V1 | PLUG-IN/POWER-SUPPLY/EU |  | /1 |
| L36851-Z2618-V2 | PLUG-IN/POWER-SUPPLY/UK |  | /1 |
| L36851-Z2618-V3 | PLUG-IN/POWER-SUPPLY/AUS |  | /1 |
| L36851-Z2618-V4 | PLUG-IN/POWER-SUPPLY/TAIWAN |  | /1 |

#) Plug-In power supply not included
*) only for handsfree carki

## Documentation

| L36859-N2600-A100-*-4-E19 | S6/USER GUIDE/UG1 | GBR/DEU/FRA/ITA | /1 |
| --- | --- | --- | --- |
| L36859-N2600-A101-*-5-W19 | S6/USER GUIDE/UG2 | DK/FIN/SWE/NOR | /1 |
| L36859-N2600-A102-*-9-K19 | S6/USER GUIDE/UG3 | GBR/KATAL/ESP/POR | /1 |
| L36859-N2600-A103-*-9-L19 | S6/USER GUIDE/UG4 | GBR/TUR/TCH/POL | /1 |
| L36859-N2600-A104-*-9-M19 | S6/USER GUIDE/UG5 | GBR/FRA/RUS/BUL | /1 |
| L36859-N2600-A105-*-9-N19 | S6/USER GUIDE/UG6 | GBR/GRI/ITA/HUN | /1 |
| L36859-N2600-A106-*-3-Q19 | S6/USER GUIDE/UG7 | GBR/FRA/NDL | /1 |
| L36859-N2600-A107-*-3-T19 | S6/USER GUIDE/UG8 | GBR/FRA/ARABIC | /1 |
| L36859-N2600-A108-*-6-C19 | S6/USER GUIDE/UG9 | GBR/CHN | /1 |
| L36859-N2600-A109-*-6-319 | S6/USER GUIDE/UG10 | GBR/DEU/FRA | /1 |
| L36859-N2600-A110-*-7-219 | S6/USER GUIDE/UG11 | ITL | /1 |
| L36859-N2600-A111-*-6-319 | S6/USER GUIDE/UG12 | GBR/DEU/FRA | /1 |
| L36859-N2600-A112-*-9-K19 | S6/USER GUIDE/UG13 | GBR/KATAL/ESP/PRT | /1 |
| L36859-N2600-A113-*-9-K19 | S6/USER GUIDE/UG14 | GBR/KATAL/ESP/PRT | /1 |
| L36859-N2600-A114-*-5-419 | S6/USER GUIDE/UG15 | NDL | /1 |
| L36859-N3600-A100-*-6-319 | S6/PCN/USER GUIDE/UG1 | DEU/GBR/FRA | /1 |